GPON OLT STICK WEB USER GUIDE

- The IP addresses for the OLT STICK are assigned via two distinct addressing schemes: static IP address assignment and dynamic IP address assignment using DHCP. By default, the device is configured for static IP address assignment upon factory shipment.

Default IP address:  192.168.1.3

Default Mask:  255.255.255.0

Default Gateway:  192.168.1.1

Default CAPWAP address:  192.168.1.100

2，The default port for web-based access is 128.

- When using the factory-set static IP, the web access address is http://192.168.1.3:128. Users need to modify the IP address of the host (computer) to the CAPWAP address or an IP address within the same network segment.

- By default, there are three levels of passwords. The username and the default password are:

Administrator username：admin, password: abcd1@

Operator username: operator, password: op1234

Viewer username: viewer, password: view123

Users can change their passwords after logging in with the administrator account. Note: Users must keep their passwords safe.

3，Modify the IP address of the OLT STICK and the address allocation mode.

- Users need to change the IP address of the host (computer) to the CAPWAP address or an IP address in the same network segment. Then, by using the default IP address to log in to the web, they can modify the static IP address of the OLT through the terminal method:

set_ipmaskgate("192.168.1.3","255.255.255.0","192.168.1.1")  //simultaneously modify the IP address, mask and gateway

IP capwap address modification: set_capwap_ip("192.16.8.2.100")

- Note: The gateway/mask/IP address must be configured correctly; otherwise, communication will be impossible.

- Modify the IP address working mode of the GPON OLT STICK

  - DHCP mode: Use the terminal command:  set_ip_addr_mode(1)

After setting, the OLT will immediately access the local DHCP server. The DHCP server can determine which OLT's IP address lease service it is based on the host-name reported by the OLT stick. The host can access the web through this OLT's IP address. If the DHCP server does not have the CAPWAP IP option, it can be set manually.

b）Static IP address mode: Use the terminal command: set_ip_addr_mode(0)

C） VLAN Operation Mode

In specific customer scenarios, if the customer has high requirements for the security of the management network, VLANs can be allocated for management services. It is recommended that the VLAN ID be greater than 3000 to avoid conflicts with the VLANs used internally by the OLT. If the customer uses VLAN, then the corresponding port VLAN settings need to be made on the switch to prevent the management host from being unable to receive and send the corresponding VLAN port messages.

Use the terminal command: set_dhcp_vlan(3000)

4, Terminal Command List

Users can access and configure the corresponding functions of the module through the terminal. The specific command contents can be obtained by using the help command.

| Command | Description | Example |
| --- | --- | --- |
| help | Help | help() |
| show_onu | Query the status of ONUs under the OLT | show_mac() |
| show_macip | Query static IP setting | show_subnet_mask() |
| show_mac | Query MAC address of OLT | show_gateway() |
| show_subnet_mask | Query Mask of OLT | show_capwapip() |
| show_gateway | Query gateway setting of OLT | show_ip() |
| show_capwapip | Query CAPWAP IP setting of OLT | show_capwapip() |
| show_ip | Query IP setting of OLT | show_ip() |
| find_onu | Check whether the ONU is on the whitelist | find_onu("WSTO12345678") |
| get_whitelst_number | Query the number of whitelist entries | get_whitelst_number() |
| add_one_onu | Add an ONU to the whitelist | add_one_onu("WSTO12345678",1) |
| clear_white_list | Clear all ONU whitelist entries in the OLT | clear_white_list() |
| del_one_onu | Delete an ONU from the whitelist | del_one_onu("WSTO12345678") |
| remove_one_onu | Remove an ONU from the whitelist | remove_one_onu("WSTO12345678") |
| get_onu_optics | Query the optical-electrical parameters of the ONU | get_onu_optics("WSTO12345678") |
| set_white_list_type | Set the gray list (0x47) / whitelist (0x57) | set_white_list_type(0x47) 或者set_white_list_type(0x57) |
| get_white_list_type | Query the current OLT working mode in the whitelist / gray list | get_white_list_type() |
| del_service_type | Delete the service template, only 2 to 5 can be deleted | del_service_type(2) |
| reset_system | Reset the OLT stick | reset_system() |
| set_ip_addr_mode | Set the OLT IP address working mode | set_ip_addr_mode(0)/set_ip_addr_mode(1) |
| get_ip_addr_mode | Query the OLT IP address working mode | get_ip_addr_mode() |
| set_capwap_ip | Set the CAPWAP IP address | set_capwap_ip("192.168.1.100") |
| set_ipmaskgate | Set IP/Mask/Gate |  |
| show_dhcp_client | Query the current OLT working IP address details | show_dhcp_client() |
| get_arp_proxy | Obtain the OLT ARP proxy enabled status | get_arp_proxy() |
| set_arp_proxy | Set the OLT arp proxy | set_arp_proxy(0)/set_arp_proxy(1) |
| PloamUserSetSN | Set the enable/disable status of the ONU |  |
| set_dhcp_vlan | Set the OLT management channel VLAN | set_dhcp_vlan(3000) |
| get_dhcp_vlan | Obtain the OLT management channel VLAN | get_dhcp_vlan() |
| get_host_name | Query the host-name of the OLT | get_host_name() |
| set_host_name | Set the host-name of the OLT | set_host_name("WSTOLT_TEST") |
| get_olt_pn | Obtain the OLT part number | get_olt_pn() |
| get_port_info | Obtain the LLDP port information of the OLT | get_port_info() |
| set_stick_pn | Set the OLT PN alias | set_stick_pn("building_one_olt") |
| get_stick_pn | Obtain the OLT PN alias | get_stick_pn() |
| onu_offline_cause | Query the reason for ONU disconnection | onu_offline_cause("WSTO12345678") |
| set_nego | Set the OLT port auto-negotiation | set_nego(0)/set_nego(1) |
| get_nego | Query the OLT port auto-negotiation | get_nego() |
| set_GEor2GE | Set the OLT port GE(1)/2GE | set_GEor2GE(1)/set_GEor2GE(0) |
| get_GEor2GE | Query the OLT port GE(1)/2GE | get_GEor2GE() |
| set_mac_addr | Set the OLT MAC address | set_mac_addr("4C4F19241112") |
| reboot_onu | Reboot the ONU | reboot_onu("WSTO12345678") |
| set_service_type | Set the service template (1 to 5) | See detailed instructions |
| get_service_type | Query the service template (1 to 5) | get_service_type(1) |